PHASE 1.04 — GIS ANALYSIS

GIS Analysis

Al Safa 2 Park — Professional GIS Layer Review & Data Gap Log

────────────────────────────────────────────────────────────

# 1. Status Overview

Professional GIS analysis (land use, building density, connectivity, slope, visibility, heat, noise, green coverage, service radius, walkability) normally requires authoritative datasets: Dubai Municipality GIS portal layers, satellite/aerial imagery, a DEM, and population/land-use shapefiles. None of these raw datasets were included in the competition files provided (only a JPG location graphic, a PDF brief, a DWG as-built, and a park design manual PDF). This document records what can legitimately be assessed now vs. what is a logged data gap.

# 2. Assessable Now (from provided materials)

| Layer | Finding | Source |
| --- | --- | --- |
| Land Use (qualitative) | Low-rise residential surrounds park on 3 sides; SZR + Al Qouz Industrial on 4th (east) side | Master-plan location image |
| Connectivity (qualitative) | Local street grid feeds the park; SZR is a hard edge/barrier on the east | Master-plan location image |
| Green Coverage (approx.) | ~0.94% of the reviewed graphic's frame flagged as green-toned pixels via color-threshold script — a rough visual proxy only, not survey-grade canopy measurement | Python image analysis (Phase 1.02) |
| Walkability (qualitative) | Park is walkable from surrounding residential streets; crossing SZR to reach metro/Al Qouz side is not pedestrian-friendly | Contextual reading |

# 3. Logged Data Gaps (require external GIS sourcing)

| Layer | Requirement |
| --- | --- |
| Building Density | Dubai Municipality building footprint GIS layer |
| Slope | Site DEM / survey contours (may exist inside the DWG once converted) |
| Visibility analysis | Requires 3D massing model of surrounding buildings |
| Heat (urban heat island) | Requires satellite thermal imagery (e.g., Landsat/Sentinel thermal bands) |
| Noise | Requires traffic count data for SZR + acoustic modeling |
| Service radius / catchment | Requires population density GIS layer for accurate isochrone modeling |

# 4. Recommendation

If genuine GIS rigor is required for the submission, the highest-value next step is: (1) convert the as-built DWG to DXF to recover any survey/contour layer; (2) optionally pull free satellite imagery for a heat/greenery cross-check — technically feasible with Python but requires external API access/credentials not yet confirmed.

This keeps the analysis honest: qualitative/contextual GIS reading is complete; quantitative GIS layers remain flagged as pending real data, not fabricated.